# Deca Lab2 Challenge Task

For the challenge we have used a circuit that multiplies two 8-bit inputs (A and B) into a 16-bit product without using the traditional shift-and-add approach. Instead, it uses a chain of 8-bit adders (each extended to 9 bits with carry), and multiplexers to form and combine partial products.

- ADD889 Block (8-bit Adder + Carry)

- Each ADD889 takes two 8-bit inputs (P and Q) and produces a 9-bit result (8 bits of sum + 1 carry bit).

- In the top‐level schematic, five of these adders are cascaded to accumulate partial products.

This ADDER889 sheet simplifies the design so that you don’t have to add a constant 0 connected to a MUX in each addition on the multiplication sheet.

- MUXs

- The multiplexers decide whether to feed in A, B, or zero/other control signals at each stage.

- By selecting which inputs go into each adder, the design effectively includes or excludes partial products without explicit shifting.

- Control Signals (C1, C2, SEL, etc.)

- These signals configure the MUXs and control whether each stage adds the relevant partial product or bypasses it.

- They also handle how the 9-bit outputs (sum + carry) get passed along from one stage to the next.

- Final Output

- After the chain of adders, the final 9-bit result of the last stage is combined (or split) into the lower and upper product bits (LSB and MSB).

- The result is ultimately presented as a 16-bit product output.

Step Simulation checking if the multiplier is working correctly:

Now we add the MUL Block to the ALU sheet:

When the ALU is instructed to perform a multiply, both operands are routed to the MUL block. The resulting 16-bit product then flows through MUX4 to become the ALU’s final output, with any relevant flags updated accordingly. MOVC is INS (4:2), MOVC = 001, the bus compare output will be 1 and hence sending the output of the multiplier to input 0 in MUX2.

Slight change to the Data Path:

This way, MOVC1 Ra, Rb means Ra := Ra x Rb

Testing the multiplier:

We upload mul3.txt onto the eep1:

MOV R0, #0x0016   // initialize op1 = 22

MOV R1, #0x0080   // initialize op2 = 128

MOVC1 R1, R0

Waveform simulation:

On clock cycle 2: instead of 0x0080, 0xFF80 is stored in R1 due to sign extension. However, this does not change the final result, 0x0B00 which is correct.

Moving onto testing 16-bit multiplication:

Mul4.txt:

// R1 and R3 are the 16 bits input

// R4 and R5 are the 32 bits output

// (31:16) (15:0) corresponding to R4 and R5 respectively

EXT 0x35

MOV R1, #123 // R1:= 0x357B

EXT 0x56

MOV R3, #109 // R3:= 0x566D

LSR R0, R1, #8 // R0 = 0x35, R1 = 0x7B

LSR R2, R3, #8 // R2 = 0x56, R3 = 0x6D

// ___R0(b)___ ___R1(a)___

// ___R2(d)___ ___R3(c)___

// note: only the last 8 bits are taken for operations

MOV R6, R1

MOVC1 R6, R3            // a x c

MOV R5, R1

MOVC1 R5, R2            // a x d

MOV R7, R0

MOVC1 R7, R3            // b x c

ADD R5, R7              // ad + bc

LSR R4, R5, #8          // add overflow of 2^8 (ad + bc)

LSL R5, R5, #8          // 2^8 (ad + bc)

ADD R5, R6              // ac + 2^8 (ad + bc)

ADC R4, #0              // add overflow of ac + 2^8 (ad + bc)

MOVC1 R2, R0            // b x d

ADD R4, R2              // bit (31:16) of the result

Therefore,

Waveform Simulation: